## Introduction

The Forest Joystick Mouse Hub allows you to connect an analog joystick and up to four (4) assistive switches to use as a USB HID Mouse or USB HID Gamepad. An additional assistive switch can be used to cycle between 3 slots and switch between Mouse mode and Gamepad mode.

## Features

Calibration button

Mode Button

Output Mode Indicator

Slot Indicator

## Usage

### Setup

- Connect an analog joystick to the Joystick port (J).

- Connect assistive switches to the Switch Input ports (S1, S2, S3, S4).

- Connect an additional assistive switch to the Slot/Mode input port (SM).

- Connect the USB-C cable to the Hub, then to the Host Device.

### Calibration

- Release the joystick to neutral.

- Press the calibration button to reset neutral position.

| Mouse Mode | Gamepad Mode |
| --- | --- |
| - Move the joystick to control the cursor. - Press the S1 button to left click. - Press the S2 button to middle click. - Press the S3 button to right click. - Press the S4 button to toggle scroll. | - Move the joystick to control the joystick. - Press the S1 button to activate Button 1. - Press the S2 button to activate Button 2. - Press the S3 button to activate Button 3. - Press the S4 button to activate Button 4. |

| Control | Short Press | Long Press |
| --- | --- | --- |
| SW Switch | Cycle Slot | Change mode |
| Mode Button | Cycle Slot | Change mode |
| Calibration Button | Neutral position reset |  |

## Host Device Compatibility

- USB HID Mouse: PC, Mac, Android Smartphone, Apple Smartphone (iOS 13+)

- USB HID Gamepad: PC, Xbox Adaptive Controller

## Specifications

| Enclosure Height | 32 mm |
| --- | --- |
| Enclosure Depth | 46 mm |
| Enclosure Length | 108 mm |

## Cleaning

The outside of the enclosure can be cleaned with a damp cloth. Do not use hot water. Do not submerge.